Procedure.

Action to be taken in the event of an oil spill.

- Stop the flow of product immediately.

- Cover drains with drain cover, matting, plastic sheeting and sand or any other suitable material to prevent product entering the drainage system.

- Notify customer/site management (if this can be done without leaving the scene).

- Notify MP Operations Team. Give as much detail as possible. A joint decision will be made as to support/assistance etc.

- Keep non-essential people away from the spill or nominate another person to do this on you behalf.

- Use your on board spill kit to soak up product.

- Do not wash product into drainage system. This will only make matters worse.

- Soak up pads can be used multiple times. Wring them out into a suitable container and reapply to the spill area.

- If using barrels to contain spilt product, make sure they are suitable and fit for purpose with bungs/lids available.

- All contaminated waste must be double bagged and disposed of in the correct manner (incineration). Arrangements must be made to have the waste removed from site as soon as possible.

- Ensure that the customer/site manager and MP Ops team are informed before you leave the site.

- An incident form and insurance form (if necessary) must be filled in completely by you upon your return to the depot.

The above bullet points are not necessarily in order.

Remember all spills are preventable.

Date:	   24/07/17

Version: 01

Review:  24/07/18

Author:  J Kaighin